Yocto Project: Building Linux for NXP i.MX8MP

1. Project Specifications

- Target Board (MACHINE): imx8mp-lpddr4-evk (i.MX 8M Plus Evaluation Kit with LPDDR4 RAM)

- Yocto Version: Kirkstone (4.0)

- Linux Kernel Version: 5.15 (Specifically 5.15-71-2.2.0)

- Distribution (DISTRO): fsl-imx-wayland (Freescale i.MX Wayland backend)

- Image Target: core-image-weston

2. Host System Setup

Before building, the host machine must be prepared with the necessary dependencies and the repo utility.

Install Host Packages

Essential Yocto Project host packages are (Ubuntu/Debian based):

Bash

$ sudo apt update

$ sudo apt-get install gawk wget git-core diffstat unzip texinfo gcc-multilib build-essential chrpath socat cpio python python3 python3-pip python3-pexpect xz-utils debianutils iputils-ping python3-git python3-jinja2 libegl1-mesa libsdl1.2-dev pylint3 xterm rsync curl zstd pzstd lz4c lz4 libssl-dev

Install Repo Utility

The repo tool is used to manage the multiple Git repositories required by Yocto.

To install the “repo” utility, perform these steps:

1. Create a bin folder in the home directory.

Bash

$ mkdir ~/bin

$ curl https://storage.googleapis.com/git-repo-downloads/repo > ~/bin/repo

$ chmod a+x ~/bin/repo

2. Add the following line to the .bashrc file to ensure that the ~/bin folder is in your PATH variable.

Bash

$ export PATH=~/bin:$PATH

3. Git account configuration:

Bash

$ git config --global user.name "Your Name"

$ git config --global user.email "Your Email"

$ git config --list

3. Source Code Management (Fetching Layers)

Initialize Repository

Fetch the NXP i.MX manifest. This points to the specific branches and revisions for the build.

Bash

$ mkdir imx-yocto-bsp

$ cd imx-yocto-bsp

$ repo init -u https://github.com/nxp-imx/imx-manifest -b imx-linux-kirkstone -m imx-5.15.71-2.2.0.xml

Sync Repositories

Download the meta-layers defined in the manifest.

Bash

$ repo sync

When this process is completed, the source code is checked out into the directory imx yocto-bsp/sources.

You can perform Repo synchronization, with the command repo sync, periodically to update to the latest code.

If errors occur during Repo initialization, try deleting the .repo directory and running the Repo initialization command again.

4. Build Configuration

The build environment must be initialized, which sets up the build directory and configuration files.

Initialization Command

The syntax for the imx-setup-release.sh script is shown below:

$ DISTRO= <distro name> MACHINE= <machine name> source imx-setup-release.sh -b <build dir>

DISTRO= <distro configuration name> is the distro, which configures the build environment, and it is stored in meta-imx/meta-sdk/conf/distro.

MACHINE= <machine configuration name> is the machine name which points to the configuration file in conf/machine in meta-freescale and meta-imx.

-b <build dir> specifies the name of the build directory created by the imx-setup-release.sh script.

Bash

$ DISTRO=fsl-imx-wayland MACHINE=imx8mp-lpddr4-evk source imx-setup-release.sh -b build

- DISTRO: Specifies the distribution policy (Wayland graphics stack).

- MACHINE: Specifies the target hardware.

- -b build: Specifies the name of the build directory.

When the script is run, it prompts the user to accept the EULA. Once the EULA is accepted, the acceptance is stored in local.conf inside each build folder and the EULA acceptance query is no longer displayed for that build folder.

After the script runs, the working directory is the one just created by the script, specified with the -b option. A conf folder is created containing the files bblayers.conf and local.conf.

The <build dir> /conf/bblayers.conf file contains all the metalayers used in the i.MX Yocto Project release.

Directory Structure (Post-Init)

- sources/: Contains all meta-layers (meta-freescale, meta-browser, meta-openembedded, etc.).

- build/: Contains configuration and build output.

  - conf/local.conf: Local build settings.

  - conf/bblayers.conf: Defines which layers are active.

5. Customization

Adding New Layers

To add functionality (e.g., Chromium browser, Games), new layers must be added to conf/bblayers.conf.
Method 1: Command Line

Bash

$ bitbake-layers add-layer ../sources/meta-games

Method 2: Manual Edit
Edit build/conf/bblayers.conf and append the path to the BBLAYERS variable.

Installing Specific Packages

To install specific software into the final image, modify build/conf/local.conf:

IMAGE_INSTALL:append = " package_name"

# Example:

IMAGE_INSTALL:append = " chromium"

NOTE: Correcting a kernel bug inside sources/poky/meta/recipes-extended/cracklib/cracklib_2.9.8.bb file.

Replace branch=master to branch=main in SRC_URI= “” line.

Modifying local.conf

Replace the contents of /imx-yocto-bsp/build-wayland/conf/local.conf with the following:

Modifying bblayers.conf

Replace the contents of /imx-yocto-bsp/build-wayland/conf/bblayers.conf with the following:

Modifying lpddr4_timing.c

Replace the imx-yocto-bsp/build-wayland/workspace/sources/u-boot-imx/board/freescale/imx8mp_evk/lpddr4_timing.c with the following file:

Modifying Kernel and U-Boot (Using Devtool)

The standard way to modify sources in Yocto without directly editing the temporary build folders are using devtool.

- Start modification:

Bash

$ devtool modify linux-imx

# OR

$ devtool modify u-boot-imx

- Edit Source:
This creates a workspace where you can edit the source code (DTS files, drivers, etc.).

- Build:

Bash

$ devtool build linux-imx

6. Device Tree & Hardware Configuration

Changes to hardware pinmuxing (GPIOs, Interfaces) are done in the Device Tree Source (.dts) files.

Key Locations

- U-Boot DTS: build-wayland/workspace/sources/u-boot-imx/arch/arm/dts/

Replace the imx8mp-evk.dts contents with the following file:

- Kernel DTS: build-wayland/workspace/sources/linux- imx/arch/arm64/boot/dts/freescale/

Replace the imx8mp-evk.dts contents with the following file:

Common Customizations

- GPIO configuration: Enable/Disable GPIOs, set initial states (Active High/Low).

- U-Boot Environment:

  - Modify CONFIG_BOOTCOMMAND to change how the kernel loads.

NOTE: In build-wayland/workspace/sources/u-boot-imx/configs/imx8mp_evk_defconfig file, make sure the following changes has been made:

- CONFIG_USB_TCP = n

- CONFIG_BOOTCOMMAND="run set-io;run distro_bootcmd;run bsp_bootcmd"

- CONFIG_DEFAULT_FDT_FILE="imx8mp-evk-delta.dtb"

NOTE: Replace the contents of build-wayland/workspace/sources/u-boot-imx/include/configs/imx8mp_evk.h with the following file:

Device Tree Integration

To ensure the custom device tree is built, add/append it in build-wayland/conf/local.conf or the machine recipe:

codeBitbake

KERNEL_DEVICETREE:append = " freescale/imx8mp-evk-delta.dtb"

NOTE: Paste the following customized .dts file in build-wayland/workspace/sources/linux-imx/arch/arm64/boot/dts/freescale/:

NOTE: Append the following in line no.18 in the path /sources/meta-freescale/wic/imx-imx-boot-bootpart.wks.in :

--label root --align 8192 --size 7000

NOTE: Replace the following file in the path /build-wayland/workspace/sources/linux-imx/drivers/phy/freescale/phy-fsl-samsung-hdmi.c :

NOTE: Replace the following file in the path /build-wayland/workspace/sources/linux-imx/drivers/tty/serial/imx.c :

NOTE: Replace the following file in the path /imx-yocto-bsp/sources/meta-delta/recipes-kernel/hid-ft260/files/hid-ft260.c

7. Building the Image

Once configured, use Bitbake to compile the image.

Source Configuration Command

Bash

$ source setup-environment build-wayland/

Build Command

Bash

$ bitbake core-image-weston

- Note: The initial build performs a lot of fetching and compiling (toolchain, kernel, rootfs) and will take significant time.

Cleaning (Troubleshooting)

If build errors occur or source changes aren't picking up:

- Clean a specific recipe:

Bash

$ bitbake -c cleanall u-boot-imx

$ bitbake -c cleanall linux-imx

$ bitbake -c cleanall core-image-weston

- Rebuild:

Bash

$ bitbake core-image-weston

8. Summary of Workflow

- Setup Host (Install packages, repo).

- Fetch Sources (repo init, repo sync).

- Configure Build (imx-setup-release.sh).

- Customize (Edit local.conf, add layers, modify DTS via devtool).

- Build (bitbake core-image-weston).

- Deploy Flash the resulting .wic.bz2 and .bin-flash_evk to the SD card/eMMC.

9. Display Mode and Boot Splash Configuration

This section records the display bring-up changes made on 22.07.2026 for the Delta board (MACHINE imx8mp-lpddr4-evk), which uses the native i.MX8M Plus HDMI path (lcdif3 -> fsl,imx8mp-hdmi).

Problem

Two symptoms were observed at boot: the psplash logo appeared displaced down and to the right, partly outside the visible screen area; and the splash flickered during boot.

Both had a single root cause. HDMI came up at the EDID preferred mode of 1920x1080, while the panel only displays the top-left 1280x800 region of it. psplash centres its image using x = (fb->width - img_width)/2, so a 1280x800 splash was placed at (320,140), outside the visible area. The un-forced per-boot EDID modeset and HDMI link-train also raced psplash drawing into the framebuffer, which produced the flicker.

The framebuffer geometry was confirmed on target with:

Bash

$ cat /sys/class/graphics/fb0/virtual_size

which returned 1920,1080 while the panel showed 1280x800.

Approaches that did not work

Setting APPEND in conf/local.conf has no effect on this machine. APPEND is only consumed by the grub-efi-cfg, syslinux, systemd-boot-cfg and uboot-extlinux-config bbclasses. This board boots from the .wks image using the U-Boot mmcargs environment variable, and UBOOT_EXTLINUX is not set, so no extlinux.conf is ever generated. The same applies to APPEND in meta-delta/conf/layer.conf.

Setting video= in the U-Boot mmcargs default environment also proved unreliable. A saved environment in eMMC shadows the compiled-in default. That environment lives at offset 0x400000 (4 MiB), which falls in the unallocated gap between imx-boot (ending near 1.99 MiB) and the boot partition (starting at 8 MiB). Because uuu transfers the image with -raw2sparse and skips all-zero blocks, that region is never rewritten and the stale environment survives every reflash.

Solution: force the mode from the kernel

The mode is now forced in the kernel command line, which makes it independent of the U-Boot environment, the flash method and the board's provisioning history. The following two lines were appended to sources/meta-delta/recipes-kernel/linux/files/custom.config:

Bash

CONFIG_CMDLINE="console=ttymxc1,115200 root=/dev/mmcblk2p2 rootwait rw quiet loglevel=0 video=HDMI-A-1:1280x800@60"

CONFIG_CMDLINE_FORCE=y

The command line string was copied verbatim from a working board's /proc/cmdline with the video= argument appended. Note that CONFIG_CMDLINE_FORCE makes the kernel ignore the bootloader command line entirely, so this string must remain in sync with the real boot arguments. The root device is fixed to the eMMC rootfs, so this kernel will not boot from an SD card without editing this line.

Kernel configuration fragment hashing fix

While applying the above it was found that edits to custom.config were being silently discarded. The fragment was previously passed to the recipe as an absolute path:

Bash

DELTA_KERNEL_DEFCONFIG:append = " ${BBAPPEND_DIR}/files/custom.config"

BitBake hashes variables, not arbitrary files on disk, so the task hash never changed when the fragment was edited. As a result sstate was reused, the kernel kept its previous .config, and stale do_rootfs and do_image_wic sstate meant that even a force-rebuilt kernel never reached the .wic. A build would report success having rebuilt nothing at all.

sources/meta-delta/recipes-kernel/linux/linux-imx_%.bbappend was therefore changed to route the fragment through SRC_URI, so that BitBake hashes its contents:

Bash

FILESEXTRAPATHS:prepend := "${THISDIR}/files:"

SRC_URI += "file://custom.config"

DELTA_KERNEL_DEFCONFIG:append = " custom.config"

Editing custom.config now correctly rebuilds the kernel and everything downstream. Any kernel option added to that file before this fix may never have taken effect, and should be verified against a built .config.

psplash placement

The splash image pdu_bg1x.png is 1280x800, matching the forced mode. Two configure options were added to sources/meta-delta/recipes-core/psplash/psplash_%.bbappend:

Bash

EXTRA_OECONF += "--enable-img-fullscreen --disable-progress-bar"

Without --enable-img-fullscreen, psplash positions the image at (fb->height * 5/6 - img_height)/2, which is negative for a full-height image and clips the top. The progress bar is disabled because its position derives from the same 5/6 split line and would otherwise be drawn over the artwork. The PROGRESS commands issued by the meta-imx psplash-basic and psplash-network services become harmless no-ops; QUIT is unaffected, so psplash-quit.service continues to work.

U-Boot mmcargs

For completeness, video=HDMI-A-1:1280x800@60 was also appended to mmcargs in include/configs/imx8mp_evk.h (line 138) of the u-boot-imx devtool workspace. With CONFIG_CMDLINE_FORCE in place this is redundant, but it is retained as a correct fallback. Note that this change is still uncaptured in the devtool workspace.

Build and verification

A single build command is sufficient; the kernel is rebuilt automatically now that the fragment is hashed correctly:

Bash

$ bitbake core-image-weston

Before flashing, confirm the artifacts were actually regenerated rather than restored from sstate. Check that the timestamp corresponds to the build just run, and that the kernel carries the forced command line:

Bash

$ ls -la --time-style=long-iso core-image-weston-imx8mp-lpddr4-evk.wic.bz2

$ grep -ac "video=HDMI-A-1:1280x800@60" Image

The image may then be flashed normally. Because the mode is compiled into the kernel, the .wic.bz2 may be used and the state of the U-Boot environment no longer matters:

Bash

$ sudo uuu -v -b emmc_all imx-boot-imx8mp-lpddr4-evk-sd.bin-flash_evk core-image-weston-imx8mp-lpddr4-evk.wic.bz2

After booting, the configuration is confirmed with:

Bash

$ cat /proc/cmdline

$ cat /sys/class/graphics/fb0/virtual_size

The command line should end with video=HDMI-A-1:1280x800@60 and the virtual size should read 1280,800. The splash then fills the panel correctly and the boot flicker is resolved.

10. FT260 GPIO Bridge Configuration

This section records the fix made on 23.07.2026 for the on-board FTDI FT260 USB-HID-to-I2C/UART bridge, whose GPIO lines were not accessible after boot.

Problem

Reading a GPIO on the FT260 gpiochip failed with an input/output error, for example:

Bash

$ gpioget gpiochip5 8

gpioget: error reading GPIO values: Input/output error

The line could only be read after manually writing 0 to the chip's uart_mode sysfs attribute, where the trailing instance number varies per board and per enumeration:

Bash

$ echo 0 > /sys/bus/hid/devices/0003:0403:6030.0002/uart_mode

Root cause

On the FT260, gpiochip line 8 (GPIO_C) is physically one of the UART RX/TX pins. These pins are available as GPIO only when the UART is off, that is when uart_mode is 0. When the UART is on (uart_mode 4), the pins are used by the UART and any GPIO access on lines 8 and 9 returns an input/output error. The driver reflects this in its enabled-GPIO mask, visible in the kernel log.

The driver writes uart_mode=0 correctly early in boot, but something on the image re-enables the UART a few seconds later, at a time that varies from boot to boot. It is not ModemManager, which is not installed, and nothing in the layers opens the /dev/ttyFT0 device. The opener is transient and releases afterwards, which is why a manual write performed later in the session takes effect and stays.

Driver is a prebuilt binary

The FT260 driver is delivered as a prebuilt kernel module. The recipe recipes-kernel/hid-ft260/hid-ft260_0.1.bb installs a pre-compiled hid-ft260.ko and does not build from source. The hid-ft260.c kept alongside it is not compiled. For this reason the fix cannot be made in the driver itself without first converting the recipe to build from source, and was applied from userspace instead.

Solution

A new recipe, recipes-core/ft260-uart-mode, installs a small systemd service that runs a continuous guard. Every two seconds the guard checks the FT260 uart_mode attribute and, if it is non-zero, writes 0 to return the pins to GPIO mode. Because the re-enable happens at an unpredictable time, a continuous guard is used rather than a single write or a fixed time window. In the steady state the guard only reads the attribute, so it does not disturb normal GPIO traffic. The service is enabled at boot and restarts automatically if it ever exits.

The recipe was added to IMAGE_INSTALL in build-wayland/conf/local.conf so it is included in the image.

Build and verification

Bash

$ bitbake core-image-weston

As with the other changes, confirm the artifacts were regenerated rather than restored from sstate before flashing, and confirm the running image after boot with its build timestamp:

Bash

$ ls -la --time-style=long-iso core-image-weston-imx8mp-lpddr4-evk.wic.bz2

$ cat /etc/timestamp

After booting the flashed image, the GPIO reads correctly without any manual step:

Bash

$ gpioget gpiochip5 8

Note that because GPIO line 8 is physically a UART pin, GPIO 8 and the /dev/ttyFT0 serial port cannot be used at the same time on this board.